Control Systems - Report

./

Version Number: 1.0

Name: Pushkar Antony

Module: Control Systems

| Ver. Rel. No. | Release Date | Prepared. By | Reviewed By | Approved By | Remarks/Revision Details |
| --- | --- | --- | --- | --- | --- |
| 1.0 | 14/04/2021 | Pushkar Antony | Shiva Kumar | Dr.Prithvi Sekhar | Individual report- Control Systems |

Document History

Contents

1(a) First Order Equation with open loop and without controller	5

Plant Description	5

Code…………	5

Math Analysis	10

Comparison Analysis:(Speed, Accuracy and stability):	10

1(b) First Order Equation with open loop and controller	10

Plant Description	10

Code….	10

Math Analysis	14

Comparison Analysis:(Speed, Accuracy and stability):	14

1(c) First Order Equation	15

Plant Description	15

Code…	15

Math Analysis	24

Comparison Analysis:(Speed, Accuracy and stability):	24

2(a) Second Order MSD Equation	24

Plant Description	24

Code…	24

Math Analysis:	30

Comparison Analysis:(Speed, Accuracy and stability):	30

2(b) Second Order MSD Equation	31

Plant Description	31

Code…	31

Math Analysis:	35

Comparison Analysis:(Speed, Accuracy and stability):	36

2(c) Second Order MSD Equation	36

Plant Description	36

Code…	37

Math Analysis:	44

Comparison Analysis:(Speed, Accuracy and stability):	45

2(d) Movement of Poles.	46

Description: Movement of poles is shown along the real and imaginary axis	46

Analysis	47

2(e) Roots of the Standard Equation	47

Code…	47

Comparison Analysis:	54

2(f) PID Analysis	55

First Order System PID Analysis	55

Second Order System PID Analysis	56

Comparison Analysis:	57

3(a) Second Order Exponential Decay system	58

Plant Description	58

Without Controller	58

Open Loop with Controller (P)	60

Open Loop with Controller (I)	61

Closed Loop- Negative feedback with Controller (D)	63

Closed Loop- Positive feedback with Controller (D)	65

Math Analysis	67

Comparison Analysis	67

4(a) 1st Order Differential Equation Model	68

4(b) 2nd Order Differential Equation Model	69

4(c) Exponential Decay- Radioactive Material Model	69

5(a) Migration of Scripts to GNU Octave	70

5(b) Migration of Model to OpenModelica	71

# 1(a) First Order Equation with open loop and without controller

## Plant Description

The Mass-damper first order system is taken as Plant. Equation: f= Bv + M v' f = force; B= coefficient of friction; M = mass ; v= velocity. Values:  B1= 0.4 M1=1000; B2= 0.5 M2= 500; B3= 1.7 M3= 340;

## Math Analysis

Independent: Time(t) Dependent: Velocity(v) and Force(f) Constant: Mass(M) and Frictional Coefficient(B)

% Roots:(-B)/M

% IVT:
% 1. For step input: 0
% 2. For impulse input: 1/M

% FVT:
% 1. For step input: 1/B
% 2. For impulse input: 0

% Time Response Results:
% Rise Time :4tau = (4M)/B; where tau = M/B

## Tool Analysis:

clc;
B1= ([0.1 0.5 1.7]);
M1=([1000 5 340]);
for i=1:3
    sys = tf([1/M1(i)],[1,B1(i)/M1(i)])
    figure(i);
    subplot(2,1,1);
    impulse(sys);
    title('Impulse Input');
    subplot(2,1,2);
    step(sys);
    title('Step Input');
    [z,p,k]= tf2zp([1/M1(i)],[1,B1(i)/M1(i)])
    figure(4);
    zplane(z,p);
    xlim([-4*1e5 2*1e5]);
    ylim([-4*1e5 2*1e5]);
    hold on;
    S = stepinfo(sys)
end

sys =
 
    0.001
  ----------
  s + 0.0001
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

  -1.0000e-04


k =

   1.0000e-03


S = 

  struct with fields:

        RiseTime: 2.1970e+04
    SettlingTime: 3.9121e+04
     SettlingMin: 9.0450
     SettlingMax: 9.9997
       Overshoot: 0
      Undershoot: 0
            Peak: 9.9997
        PeakTime: 1.0546e+05


sys =
 
    0.2
  -------
  s + 0.1
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

   -0.1000


k =

    0.2000


S = 

  struct with fields:

        RiseTime: 21.9701
    SettlingTime: 39.1207
     SettlingMin: 1.8090
     SettlingMax: 1.9999
       Overshoot: 0
      Undershoot: 0
            Peak: 1.9999
        PeakTime: 105.4584


sys =
 
  0.002941
  ---------
  s + 0.005
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

   -0.0050


k =

    0.0029


S = 

  struct with fields:

        RiseTime: 439.4013
    SettlingTime: 782.4149
     SettlingMin: 0.5321
     SettlingMax: 0.5882
       Overshoot: 0
      Undershoot: 0
            Peak: 0.5882
        PeakTime: 2.1092e+03

## Comparison Analysis:

%Stability: System 2 is more stable as roots are more negetive.
%Accuracy:  Settling time of System 2 is lowest so System 2 is Accurate.
%Speed:  Rise time of System 2 is lowest so System 2 has the highest speed.

# 1(b) First Order Equation with open loop and controller

## Plant Description Plant Description

The Mass-damper first order system is taken as Plant. Equation: f= Bv + M v' f = force; B= coefficient of friction; M = mass ; v= velocity. Values:  B1= 0.4 M1=1000; B2= 0.5 M2= 500; B3= 1.7 M3= 340;

## Math Analysis

Independent: Time(t) Dependent: Velocity(v) and Force(f) Constant: Mass(M) and Frictional Coefficient(B)

% Roots:(-B)/M

% Poles and Zero Calculation:


% IVT:
% 1. For step input: 0
% 2. For impulse input: 1/M

% FVT:
% 1. For step input: 1/B
% 2. For impulse input: 0

% Time Response Results:
% Rise Time :4tau = (4M)/B; where tau = M/B

## Tool Analysis:

clc;
B1= 0.5;
M1= 5;
P = 2;

 sys = tf([P/M1],[1,B1/M1])
 subplot(4,3,1);
 impulse(sys);
 title('Impulse Input for k');
 subplot(4,3,2);
 step(sys);
 title('Step Input for k');
 subplot(4,3,3);
 [z,p,k]= tf2zp([P/M1],[1,B1/M1])
 pzmap(sys)
 subplot(4,3,10);
 bode(sys)

 hold on;
 S = stepinfo(sys)

sys = tf([P/M1],[1,B1/M1,0])
subplot(4,3,4);
impulse(sys);
title('Impulse Input for 1/s');
subplot(4,3,5);
step(sys);
title('Step Input for 1/s');
subplot(4,3,6);
[z,p,k]= tf2zp([P/M1],[1,B1/M1,0])
pzmap(sys)
 subplot(4,3,11);
 bode(sys)
hold on;
S = stepinfo(sys)
sys = tf([P/M1,0],[1,B1/M1])
subplot(4,3,7);
impulse(sys);
title('Impulse Input for s');
subplot(4,3,8);
step(sys);
title('Step Input for s');
subplot(4,3,9);
[z,p,k]= tf2zp([P/M1,0],[1,B1/M1])
pzmap(sys)
 subplot(4,3,12);
 bode(sys)
hold on;
S = stepinfo(sys)

sys =
 
    0.4
  -------
  s + 0.1
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

   -0.1000


k =

    0.4000


S = 

  struct with fields:

        RiseTime: 21.9701
    SettlingTime: 39.1207
     SettlingMin: 3.6180
     SettlingMax: 3.9999
       Overshoot: 0
      Undershoot: 0
            Peak: 3.9999
        PeakTime: 105.4584


sys =
 
      0.4
  -----------
  s^2 + 0.1 s
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

         0
   -0.1000


k =

    0.4000


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


sys =
 
   0.4 s
  -------
  s + 0.1
 
Continuous-time transfer function.


z =

     0


p =

   -0.1000


k =

    0.4000


S = 

  struct with fields:

        RiseTime: 21.9701
    SettlingTime: 39.1207
     SettlingMin: 1.0521e-05
     SettlingMax: 0.0382
       Overshoot: 0
      Undershoot: 7.2058e+17
            Peak: 0.4000
        PeakTime: 0

## Comparison Analysis:

Propotional controller will have no effect on Rising time and setting time of the system hence there response parameters remains same for all values of p and only the amplitude of the system get affected by P times.

## %Integrator controller is used to eliminate steady state error from the
%system , but as a pole is added system is going from stable to marginally
%stable.
%Differentiator control on system adds a zero to the system and makes
%unstable system stable in our case

%Stability: Both System 1 and System 3 are stable as roots are negative and have same system responses.
%Accuracy:  Settling time of System 1 and System 3  is lowest so they are Accurate.
%Speed:  Rise time of System 1 and System 3 is lowest so they are speed system.

# First Order Equation CL

## Plant Description

The Mass-damper first order system is taken as Plant. Equation: f= Bv + M v' f = force; B= coefficient of friction; M = mass ; v= velocity. Values:  B1= 0.4 M1=1000; B2= 0.5 M2= 500; B3= 1.7 M3= 340;

## Math Analysis

Independent: Time(t) Dependent: Velocity(v) and Force(f) Constant: Mass(M) and Frictional Coefficient(B)

% Roots:(-B)/M

% Poles and Zero Calculation:

% IVT:
% 1. For step input: 0
% 2. For impulse input: 1/M

% FVT:
% 1. For step input: 1/B
% 2. For impulse input: 0

% Time Response Results:
% Rise Time :4tau = (4M)/B; where tau = M/B

## Tool Analysis:

%Negative Feedback using gain input
clc;
%Stable P control
B1= 0.5;
M1= 5;
P = 2;

sys = tf([P],[M1,B1+1])
subplot(4,4,1);
impulse(sys);
title('Impulse Input for k');
subplot(4,4,2);
step(sys);
title('Step Input for k');
subplot(4,4,3);
[z,p,k]= tf2zp([P],[M1,B1+1])
pzmap(sys)
subplot(4,4,4)
bode(sys)
margin(sys)
[Gm,Pm,Wcg,Wcp] = margin(sys)
hold on;
S = stepinfo(sys)

%Unstable P controll
B2= -2;
M2= 5;
P2 = 2;

sys = tf([P2],[M2,B2+1])
subplot(4,4,5);
impulse(sys);
title('Impulse Input for k-Unstable');
subplot(4,4,6);
step(sys);
title('Step Input for k-Unstable');
subplot(4,4,7);
[z,p,k]= tf2zp([P2],[M2,B2+1])
pzmap(sys)
subplot(4,4,8)
bode(sys)
margin(sys)
[Gm,Pm,Wcg,Wcp] = margin(sys)
hold on;
S = stepinfo(sys)

%Positive Feedback using integral input
B3= 0.8;
M3= 5;


sys = tf([1],[M3,B3-1,0])
subplot(4,4,9);
impulse(sys);
title('Step Input for Positive feedback');
subplot(4,4,10);
step(sys);
title('Step Input for Positive feedback');
subplot(4,4,11);
[z,p,k]= tf2zp([1],[M3,B3-1,0])
pzmap(sys)
subplot(4,4,12)
bode(sys)
margin(sys)
[Gm,Pm,Wcg,Wcp] = margin(sys)
hold on;
S = stepinfo(sys)

% Positive Feedback using differentiator input
B4= 0.8;
M4= 5;


sys = tf([1,0],[M4,B4-1])
subplot(4,4,13);
impulse(sys);
title('Step Input for Positive feedback');
subplot(4,4,14);
step(sys);
title('Step Input for Positive feedback');
subplot(4,4,15);
[z,p,k]= tf2zp([1,0],[M4,B4-1])
pzmap(sys)
subplot(4,4,16)
bode(sys)
margin(sys)
[Gm,Pm,Wcg,Wcp] = margin(sys)
hold on;
S = stepinfo(sys)

sys =
 
      2
  ---------
  5 s + 1.5
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

   -0.3000


k =

    0.4000


Gm =

   Inf


Pm =

  138.5925


Wcg =

   NaN


Wcp =

    0.2646


S = 

  struct with fields:

        RiseTime: 7.3234
    SettlingTime: 13.0402
     SettlingMin: 1.2060
     SettlingMax: 1.3333
       Overshoot: 0
      Undershoot: 0
            Peak: 1.3333
        PeakTime: 35.1528


sys =
 
     2
  -------
  5 s - 1
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

    0.2000


k =

    0.4000


Gm =

    0.5000


Pm =

   59.9993


Wcg =

     0


Wcp =

    0.3464


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


sys =
 
        1
  -------------
  5 s^2 - 0.2 s
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

         0
    0.0400


k =

    0.2000

Warning: The closed-loop system is
unstable. 

Gm =

   Inf


Pm =

   -5.1214


Wcg =

   Inf


Wcp =

    0.4463


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


sys =
 
      s
  ---------
  5 s - 0.2
 
Continuous-time transfer function.


z =

     0


p =

    0.0400


k =

    0.2000

Warning: The closed-loop system is
unstable. 

Gm =

   Inf


Pm =

   Inf


Wcg =

   NaN


Wcp =

   NaN


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf

## Comparison Analysis:

%Negetive feedback system:
%for negetive feedback system poles lie in -0.3
%and hence the system is stable, and when the controller is added to the
%system rasie time and settiling time of the response is reduced and
%system is becoming more fast and stable.

%Positive feedback:
%In positive feedback system the roots are lying in the
%right hand side of the s plane and the system is unstable.
%Bode Plot:
%phase margin-139degree and gain margin-infinite
%when the phase of the loop gain goes below -180 degree then gain margin is
%infinite Wcp=0.264.
%phase margin:-60 degree and gain margin:-6.02dB
%phase margin is infinite because the magnitude plot goes below 0dB at all
%frequencies this indicates the system will have trouble tracking various
%reference signal without excessive error.
% Rise time of Sys2 is lowest so Sys2 has the highest speed.]
% Settling time of Sys2 is lowest so Sys2 is most stable.
%the gain margin is infinite for all values if Kp since the gain is zero at
%gain cross over frequency

# 2(a) Second Order MSD Equation

## Plant Description

The Mass-damper Spring Second order system is taken as Plant. It is used in as suspension.

% Equation: Mx''(t)+ Bx'(t) + Kx(t)= Kf(t).
% f = force; B= coefficient of friction; M = mass ; v= velocity; k=spring constant.
% Values: K1= 0.9 B1= 0.4 M1=1000 Wn=0.03 ; K2= 1 B2= 0.5 M2= 500 Wn=0.44; K3= 3 B3= 1.7 M3= 340 Wn=0.09;

## Math Analysis:

Independent: Time(t) Dependent: Velocity(v) and Force(f) Constant: Mass(M), Frictional Coefficient(B), Spring constant(K)

% Roots:((-B/M)+-sqrt(sq(B/M)-4K/M))/2


% IVT:
% 1. For step input: 0
% 2. For impulse input: 0

% FVT:
% 1. For step input: 1
% 2. For impulse input: K/M

## Tool Analysis:

B1= ([0.1 0.5 1.7]);
M1=([1000 5 340]);M1=([1000 5 340]);
K1 = ([0.9 1 3]);
for i=1:3
    sys = tf([K1(i)/M1(i)],[1,B1(i)/M1(i),K1(i)/M1(i)])
    figure(i);
    subplot(2,1,1);
    impulse(sys);
    title('Impulse Input');
    subplot(2,1,2);
    step(sys);
    title('Step Input');
    [z,p,k]= tf2zp([K1(i)/M1(i)],[1,B1(i)/M1(i),K1(i)/M1(i)])
    figure(4);
    zplane(z,p);
    xlim([-5*1e5 3*1e5]);
    ylim([-5*1e5 3*1e5]);
    hold on;
    S = stepinfo(sys)
end

sys =
 
          0.0009
  -----------------------
  s^2 + 0.0001 s + 0.0009
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

  -0.0001 + 0.0300i
  -0.0001 - 0.0300i


k =

   9.0000e-04

License checkout failed.
License Manager Error -10
Your license for Signal_Toolbox has expired. 
If you are not using a trial license contact your License Administrator to obtain an updated license. 
Otherwise, contact your Sales Representative for a trial extension.

Troubleshoot this issue by visiting: 
<a href="https://www.mathworks.com/support/lme/R2020b/10">https://www.mathworks.com/support/lme/R2020b/10</a>

Diagnostic Information:
Feature: Signal_Toolbox 
License path: C:\Users\99003728\AppData\Roaming\MathWorks\MATLAB\R2020b_licenses;C:\Program Files\MATLAB\R2020b\licenses\license.dat;C:\Program Files\MATLAB\R2020b\licenses\trial_8681003_R2020b.lic 
Licensing error: -10,32.

Error in sec_order_sys_MSD (line 52)
    zplane(z,p);

## Comparison Analysis:

Time Response Results:

%system
% K1= 0.9 B1= 0.4 M1=1000
%RiseTime: 34.7791
%SettlingTime: 7.8226e+04
%percentage Overshoot: 99.4778
%PeakTime: 104.7198


%K2= 1 B2= 0.5 M2= 500
%RiseTime:2.5448
%SettlingTime: 78.1524
%percentage Overshoot: 70.2118
%PeakTime: 70.2118

%K3= 3 B3= 1.7 M3= 340
%RiseTime:1.5426e+03
%SettlingTime: 0.1540
%percentage Overshoot: 70.2118
%PeakTime: 33.4448
%Speed: System 2 is having low raise time and is therefore speed system.
%Stability: Settling time of system of system 3 is less and therefore is
%the stable system.
%Accuracy: Settling time of system of system 3 is less and therefore is
%more accurate.

# 2(b) Second Order MSD Equation

## Plant Description

The Mass-damper Spring Second order system is taken as Plant. It is used in as suspension.

% Equation: Mx''(t)+ Bx'(t) + Kx(t)= Kf(t).
% f = force; B= coefficient of friction; M = mass ; v= velocity; k=spring constant.
% Values: K1= 0.9 B1= 0.4 M1=1000 Wn=0.03 ; K2= 1 B2= 0.5 M2= 500 Wn=0.44; K3= 3 B3= 1.7 M3= 340 Wn=0.09;

## Math Analysis:

Independent: Time(t) Dependent: Velocity(v) and Force(f) Constant: Mass(M), Frictional Coefficient(B), Spring constant(K)

% Roots:((-B/M)+-sqrt(sq(B/M)-4K/M))/2

% Poles and Zero Calculation:


% IVT:
% 1. For step input: 0
% 2. For impulse input: 0

% FVT:
% 1. For step input: 1
% 2. For impulse input: K/M

% Time Response Results:

% K1= 1 B1= 0.5 M1= 500
% P controller:-

% Rise Time :2.5448
% settling time:78.1524
% Overshoot:70.2118
% PeakTime: 7.0248

% I controller:-

% Rise Time :NaN
% settling time:NaN
% Overshoot:NaN
% PeakTime:Inf

% D controller:-

% Rise Time :0
% settling time:81.5509
% Overshoot: Inf
% PeakTime: 3.5124

## Tool Analysis:

B1= 0.5
M1= 5;
K1 =1;
P=5;
sys = tf([P*K1/M1],[1,B1/M1,K1/M1])
subplot(4,3,1);
impulse(sys);
title('Impulse Input for k');
subplot(4,3,2);
step(sys);
title('Step Input for k');
subplot(4,3,3);
[z,p,k]= tf2zp([P*K1/M1],[1,B1/M1,K1/M1])
pzmap(sys)
subplot(4,3,10);
bode(sys)
hold on;
S = stepinfo(sys)

sys = tf([P*K1/M1],[1,B1/M1,K1/M1,0])
subplot(4,3,4);
impulse(sys);
title('Impulse Input for 1/s');
subplot(4,3,5);
step(sys);
title('Step Input for 1/s');
subplot(4,3,6);
[z,p,k]= tf2zp([P*K1/M1],[1,B1/M1,K1/M1,0])
pzmap(sys)
subplot(4,3,11);
bode(sys)
hold on;
S = stepinfo(sys)

sys = tf([P*K1/M1,0],[1,B1/M1,K1/M1])
subplot(4,3,7);
impulse(sys);
title('Impulse Input for s');
subplot(4,3,8);
step(sys);
title('Step Input for s');
subplot(4,3,9);
[z,p,k]= tf2zp([P*K1/M1,0],[1,B1/M1,K1/M1])
pzmap(sys)
subplot(4,3,12);
bode(sys)
hold on;
S = stepinfo(sys)

B1 =

    0.5000


sys =
 
          1
  -----------------
  s^2 + 0.1 s + 0.2
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

  -0.0500 + 0.4444i
  -0.0500 - 0.4444i


k =

     1


S = 

  struct with fields:

        RiseTime: 2.5448
    SettlingTime: 78.1524
     SettlingMin: 2.5361
     SettlingMax: 8.5106
       Overshoot: 70.2118
      Undershoot: 0
            Peak: 8.5106
        PeakTime: 7.0248


sys =
 
            1
  ---------------------
  s^3 + 0.1 s^2 + 0.2 s
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

   0.0000 + 0.0000i
  -0.0500 + 0.4444i
  -0.0500 - 0.4444i


k =

     1


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


sys =
 
          s
  -----------------
  s^2 + 0.1 s + 0.2
 
Continuous-time transfer function.


z =

     0


p =

  -0.0500 + 0.4444i
  -0.0500 - 0.4444i


k =

     1


S = 

  struct with fields:

        RiseTime: 0
    SettlingTime: 81.5509
     SettlingMin: -1.3280
     SettlingMax: 1.8877
       Overshoot: Inf
      Undershoot: Inf
            Peak: 1.8877
        PeakTime: 3.5124

## Comparison Analysis:

%P-Controller:
%Using p controller the system responses doesnt change and only magnitude
%of response is increased and as the poles are complex conjugate and negetive hence system is stable
%I-controller:
%Using I controller a pole is added at zero and  makes the system
%marginally stable so we dont get raise time,settling time and overshoot.
%D-controller:
%Using differentiator controller a zero is added and as we know zero is
%root of the numerator of the transfer function and it introduces a
%pronounced peak to peak system response and hence the systems peak
%overshoot may increase.

%Speed: System 3 is having low raise time and is therefore speed system.
%Stability: Settling time of system of system 1 is less and therefore is
%the stable system.
%Accuracy: Settling time of system of system 1 is less and therefore is
%more accurate.

# 2(c) Second Order MSD Equation

## Plant Description

The Mass-damper Spring Second order system is taken as Plant. It is used in as suspension.

% Equation: Mx''(t)+ Bx'(t) + Kx(t)= Kf(t).
% f = force; B= coefficient of friction; M = mass ; v= velocity; k=spring constant.
% Values: K1= 0.9 B1= 0.4 M1=1000 Wn=0.03 ; K2= 1 B2= 0.5 M2= 500 Wn=0.44; K3= 3 B3= 1.7 M3= 340 Wn=0.09;

## Math Analysis:

Independent: Time(t) Dependent: Velocity(v) and Force(f) Constant: Mass(M), Frictional Coefficient(B), Spring constant(K)

% Roots:((-B/M)+-sqrt(sq(B/M)-4K/M))/2

% Poles and Zero Calculation:


% IVT:
% 1. For step input: 0
% 2. For impulse input: 0

% FVT:
% 1. For step input: 1
% 2. For impulse input: K/M

## Tool Analysis:

clc;
%For negative feedback
B1= 0.5
M1= 5;
K1 =1;
P=5;

sys = tf([P*K1],[M1,B1,2*K1])
subplot(4,4,1);
impulse(sys);
title('Impulse Input for k');
subplot(4,4,2);
step(sys);
title('Step Input for k');
subplot(4,4,3);
[z,p,k]= tf2zp([P*K1],[M1,B1,2*K1])
pzmap(sys)
subplot(4,4,4)
bode(sys)
margin(sys)
[Gm,Pm,Wcg,Wcp] = margin(sys)

hold on;
S = stepinfo(sys)

B2= -9
M2= 5;
K2=1;
P2=5;

sys = tf([P2*K2],[M2,B2,2*K2])
subplot(4,4,5);
impulse(sys);
title('Impulse Input for k- Unstable');
subplot(4,4,6);
step(sys);
title('Step Input for k- Unstable');
subplot(4,4,7);
[z,p,k]= tf2zp([P2*K2],[M2,B2,2*K2])
pzmap(sys)
subplot(4,4,8)
bode(sys)
margin(sys)
[Gm,Pm,Wcg,Wcp] = margin(sys)
hold on;
S = stepinfo(sys)

% For Positive feedback using I & D

B3= 9
M3= 5;
K3=1;


sys = tf([K3],[M3,B3,0,0])
subplot(4,4,9);
impulse(sys);
title('Impulse Input for Positive feedback 1/s ');
subplot(4,4,10);
step(sys);
title('Step Input for Positive feedback 1/s');
subplot(4,4,11);
[z,p,k]= tf2zp([K3],[M3,B3,0,0])
pzmap(sys)
subplot(4,4,12)
bode(sys)
margin(sys)
[Gm,Pm,Wcg,Wcp] = margin(sys)
hold on;
S = stepinfo(sys)


B4= 9
M4= 5;
K4=1;


sys = tf([K4,0],[M4,B4,0])
subplot(4,4,13);
impulse(sys);
title('Impulse Input for Positive feedback s ');
subplot(4,4,14);
step(sys);
title('Step Input for Positive feedback s');
subplot(4,4,15);
[z,p,k]= tf2zp([K4,0],[M4,B4,0])
pzmap(sys)
subplot(4,4,16)
bode(sys)
margin(sys)
[Gm,Pm,Wcg,Wcp] = margin(sys)
hold on;
S = stepinfo(sys)

B1 =

    0.5000


sys =
 
          5
  -----------------
  5 s^2 + 0.5 s + 2
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

  -0.0500 + 0.6305i
  -0.0500 - 0.6305i


k =

     1


Gm =

   Inf


Pm =

    6.7782


Wcg =

   Inf


Wcp =

    1.1803


S = 

  struct with fields:

        RiseTime: 1.7526
    SettlingTime: 75.6433
     SettlingMin: 0.9814
     SettlingMax: 4.4486
       Overshoot: 77.9429
      Undershoot: 0
            Peak: 4.4486
        PeakTime: 4.9673


B2 =

    -9


sys =
 
         5
  ---------------
  5 s^2 - 9 s + 2
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

    1.5403
    0.2597


k =

     1

Warning: The closed-loop system is
unstable. 

Gm =

   Inf


Pm =

  -95.4008


Wcg =

   Inf


Wcp =

    0.5531


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


B3 =

     9


sys =
 
        1
  -------------
  5 s^3 + 9 s^2
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

         0
         0
   -1.8000


k =

    0.2000

Warning: The closed-loop system is
unstable. 

Gm =

     0


Pm =

  -10.4065


Wcg =

     0


Wcp =

    0.3306


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


B4 =

     9


sys =
 
       s
  -----------
  5 s^2 + 9 s
 
Continuous-time transfer function.


z =

     0


p =

         0
   -1.8000


k =

    0.2000

Warning: The closed-loop system is
unstable. 

Gm =

   Inf


Pm =

   Inf


Wcg =

   NaN


Wcp =

   NaN


S = 

  struct with fields:

        RiseTime: 1.2206
    SettlingTime: 2.1734
     SettlingMin: 0.1005
     SettlingMax: 0.1111
       Overshoot: 0
      Undershoot: 0
            Peak: 0.1111
        PeakTime: 5.8588

## Comparison Analysis:

%Negetive feedback with P-controller:
%Using p controller the system responses doesnt change and only magnitude
%of response is increased and as the poles are complex conjugate and negetive hence system is stable

%Positive feedback with P-controller:
%here poles are complex conjugate and are lying on the right hand side of s
%plane and hence the system is unstable.

%Positive feedback with I_controller:
%Using I controller a pole is added at zero and  makes the system
%unstable as %GM=9;PM=negetive hence stability of system is affetced and becomes a
%unstable system.

%positive feedback with D-controller:
%Using differentiator controller a zero is added at the origin and
%therefore the system becomes stable.


%Speed: System 1 is having low raise time and is therefore speed system.
%Stability: Settling time of system of system 4 is less and therefore is
%the stable system.
%Accuracy: Settling time and raise time are less of system 4 is less and therefore is
%more accurate.

# 2(d) Movement of Poles.

## Description: Here the movement of poles is shown along the real and imaginary axis .

poles = [-10+20i -10-20i -5+10i -5-10i -10+10i -10-10i  3+10i 3-10i -5+0i +5+0i -10+0i +10-0i ];

zeros = [0 0];

gain = 0.9;

s=zpk(zeros,poles,gain);

pzplot(s)

[wn,zeta] = damp(s)

wn =

    5.0000
    5.0000
   10.0000
   10.0000
   10.4403
   10.4403
   11.1803
   11.1803
   14.1421
   14.1421
   22.3607
   22.3607


zeta =

    1.0000
   -1.0000
    1.0000
   -1.0000
   -0.2873
   -0.2873
    0.4472
    0.4472
    0.7071
    0.7071
    0.4472
    0.4472

## Analysis :

1. If we move the roots along Wn, then frequency will increase and overshoot remains same. 2. If we move the poles along jw axis, then overshoot increases and frequency also increases. 3. If we move along zetaWn axis or x-axis. 3a. If we move to right hand side then overshoot increases and frequency decreases. 3b. If we move to left hand side of s-plane overshoot decreases and frequency increases.

# 2(e) Roots of the Standard Equation

## Code

clc;
zeta=1;
TF=tf([1],[1,(2*zeta),1])
sys = tf([1],[1,(2*zeta),1])
figure
subplot(2,3,1)
S = stepinfo(sys)
[z,p,k]= tf2zp([1],[1,(2*zeta),1])
zplane(z,p)

zeta=0.7 ;
TF=tf([1],[1,(2*zeta),1])
sys = tf([1],[1,(2*zeta),1])
subplot(2,3,2)
S = stepinfo(sys)
[z,p,k]= tf2zp([1],[1,(2*zeta),1])
zplane(z,p)

zeta=2;
TF=tf([1],[1,(2*zeta),1])
sys = tf([1],[1,(2*zeta),1])
subplot(2,3,3)
S = stepinfo(sys)
[z,p,k]= tf2zp([1],[1,(2*zeta),1])
zplane(z,p)

zeta=-1.85;
TF=tf([1],[1,(2*zeta),1])
sys = tf([1],[1,(2*zeta),1])
subplot(2,3,4)
S = stepinfo(sys)
[z,p,k]= tf2zp([1],[1,(2*zeta),1])
zplane(z,p)

zeta=-0.4;
TF=tf([1],[1,(2*zeta),1])
sys = tf([1],[1,(2*zeta),1])
subplot(2,3,5)
S = stepinfo(sys)
[z,p,k]= tf2zp([1],[1,(2*zeta),1])
zplane(z,p)

zeta=-2.45;
TF=tf([1],[1,(2*zeta),1])
sys = tf([1],[1,(2*zeta),1])
subplot(2,3,6)
S = stepinfo(sys)
[z,p,k]= tf2zp([1],[1,(2*zeta),1])
zplane(z,p)

TF =
 
        1
  -------------
  s^2 + 2 s + 1
 
Continuous-time transfer function.


sys =
 
        1
  -------------
  s^2 + 2 s + 1
 
Continuous-time transfer function.


S = 

  struct with fields:

        RiseTime: 3.3579
    SettlingTime: 5.8339
     SettlingMin: 0.9000
     SettlingMax: 0.9994
       Overshoot: 0
      Undershoot: 0
            Peak: 0.9994
        PeakTime: 9.7900


z =

  0×1 empty double column vector


p =

    -1
    -1


k =

     1


TF =
 
         1
  ---------------
  s^2 + 1.4 s + 1
 
Continuous-time transfer function.


sys =
 
         1
  ---------------
  s^2 + 1.4 s + 1
 
Continuous-time transfer function.


S = 

  struct with fields:

        RiseTime: 2.1268
    SettlingTime: 5.9789
     SettlingMin: 0.9001
     SettlingMax: 1.0460
       Overshoot: 4.5986
      Undershoot: 0
            Peak: 1.0460
        PeakTime: 4.4078


z =

  0×1 empty double column vector


p =

  -0.7000 + 0.7141i
  -0.7000 - 0.7141i


k =

     1


TF =
 
        1
  -------------
  s^2 + 4 s + 1
 
Continuous-time transfer function.


sys =
 
        1
  -------------
  s^2 + 4 s + 1
 
Continuous-time transfer function.


S = 

  struct with fields:

        RiseTime: 8.2308
    SettlingTime: 14.8789
     SettlingMin: 0.9017
     SettlingMax: 0.9993
       Overshoot: 0
      Undershoot: 0
            Peak: 0.9993
        PeakTime: 27.3269


z =

  0×1 empty double column vector


p =

   -3.7321
   -0.2679


k =

     1


TF =
 
         1
  ---------------
  s^2 - 3.7 s + 1
 
Continuous-time transfer function.


sys =
 
         1
  ---------------
  s^2 - 3.7 s + 1
 
Continuous-time transfer function.


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


z =

  0×1 empty double column vector


p =

    3.4064
    0.2936


k =

     1


TF =
 
         1
  ---------------
  s^2 - 0.8 s + 1
 
Continuous-time transfer function.


sys =
 
         1
  ---------------
  s^2 - 0.8 s + 1
 
Continuous-time transfer function.


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


z =

  0×1 empty double column vector


p =

   0.4000 + 0.9165i
   0.4000 - 0.9165i


k =

     1


TF =
 
         1
  ---------------
  s^2 - 4.9 s + 1
 
Continuous-time transfer function.


sys =
 
         1
  ---------------
  s^2 - 4.9 s + 1
 
Continuous-time transfer function.


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf


z =

  0×1 empty double column vector


p =

    4.6866
    0.2134


k =

     1

## Comparison Analysis:

% 1st value lise on negative x axis means: Critically-damped case & stable
% 2nd value lise in 2nd & 3rd quadrant means: Under-damp case & stable
% 3rd value lise on negative x axis means: Overdamped case & stable
% 4th value lise on positive x axis means: unstable
% 5th value lise on 1st & 4th quadrant means: unstable
% 6th value lise on positive x axis means: unstable

# 2(f) PID Analysis Second Order PID controller:

Author: Mohammed Ijaz PS Number: 99003728 Date: 7th April 2021. Version: Matlab 2020

Second Order System PID	1

Analysis of Second Order system with PID controllers	2

First Order System PID Code	3

Analysis of First Order system with PID controllers	4

## Second Order System PID

B1= 0.5
M1= 5;
K1 =1;
P1=5;

sys1 = tf([P1*K1],[M1,B1,2*K1])
pidTuner(sys1)

B1 =

    0.5000


sys1 =
 
          5
  -----------------
  5 s^2 + 0.5 s + 2
 
Continuous-time transfer function.

## Analysis of Second Order system with PID controllers

% PI Controller
% Ideal values given by the pid tuner:
% Kp =0 Ki =0.0174
% Rise time=50.4s
% Settling time= 93.4s
% Overshoot= 0.00872%


% To make 0% overshoot we are decreasing the speed of the system


% The new values are given below:
% Kp =0 Ki =0.015
% Rise time=51.1s
% Settling time= 94.3s
% Overshoot= 0


% PD Controller
% Ideal values given by the pid tuner:
% Kp =27.35 Kd =6.251
% Rise time=0.017s
% Settling time= 1.35s
% Overshoot= 24.7%


% To make overshoot 0% we are increasing the speed.
% The new values are given below:


% Kp =26.97 Kd =6.34
% Rise time=0.0179s
% Settling time= 0.13s
% Overshoot= 0%


% PID Controller
% Ideal values given by the pid tuner:


% This is the best possible reponse given by the system i.e. we cannot
% decrease the overshoot further more.


% Kp =26.97 Kd =6.34 Ki =0.68
% Rise time=0.495s
% Settling time= 9.3s
% Overshoot= 12.4%

## First Order System PID Code

B2= 0.5;
M2= 5;
P2 = 2;


sys2 = tf([P2],[M2,B2+1])
pidTuner(sys2)

sys2 =
 
      2
  ---------
  5 s + 1.5
 
Continuous-time transfer function.

## Analysis of First Order system with PID controllers

PI Controller Ideal values given by the pid tuner: Kp =0.781 Ki =0.70 Rise time=2.7s Settling time= 9.87s Overshoot= 13.8%

% Changing the values of Kp and ki to get the best possible stability of
% the system i.e decreasing the overshoot.


% The new values are given below:
% Kp =1.25 Ki =0.46
% Rise time=3.59s
% Settling time= 5.39s
% Overshoot= 1.33%


% PD Controller
% Ideal values given by the pid tuner:
% Kp =53.18 Kd =0
% Rise time=0.102s
% Settling time= 0.181s
% Overshoot= 0%


% As we are getting the overshoot 0% so we don't need to change any
% parameters.


% PID Controller
% Ideal values given by the pid tuner:


% Kp =1.07 Kd =0 Ki =0.53
% Rise time=3.04s
% Settling time= 10.6s
% Overshoot= 6.08%


% The above parameters gives the best response for the system so we don't
% need to change any parameters.

# 3(a) Second Order Exponential Decay system

## Plant Description

It is a exponential decay system of a radioactive material Equation- dM/dt=-kA(e^-kt) M=mass, k=constant, A= non zero constant, t=time Values- k=0.14, A=200

## Without Controller

clc;
k= 0.14;
A= 200;
sys = tf([-k*A],[1,k,0])
figure(1);
subplot(3,1,1);
impulse(sys);
title('Impulse Input');
subplot(3,1,2);
step(sys);
title('Step Input');
[z,p,k] = tf2zp([-k*A],[1,k,0])
subplot(3,1,3);
zplane(z,p);
S = stepinfo(sys)

sys =
 
      -28
  ------------
  s^2 + 0.14 s
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

         0
   -0.1400


k =

  -28.0000


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf

## Open Loop with Controller (P)

P= 2;
sys = tf([P*(-k)*A],[1,k,0])
figure(2);
subplot(3,1,1);
impulse(sys);
title('Impulse Input');
subplot(3,1,2);
step(sys);
title('Step Input');
[z,p,k] = tf2zp([P*(-k)*A],[1,k,0])
subplot(3,1,3);
zplane(z,p);
S = stepinfo(sys)

sys =
 
   1.12e04
  ----------
  s^2 - 28 s
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

         0
   28.0000


k =

   1.1200e+04


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf

## Open Loop with Controller (I)

sys = tf([(-k)*A],[1,k,0,0])
figure(3);
subplot(3,1,1);
impulse(sys);
title('Impulse Input');
subplot(3,1,2);
step(sys);
title('Step Input');
[z,p,k] = tf2zp([-k*A],[1,k,0,0])
subplot(3,1,3);
zplane(z,p);
S = stepinfo(sys)

sys =
 
      -2.24e06
  -----------------
  s^3 + 1.12e04 s^2
 
Continuous-time transfer function.


z =

  0×1 empty double column vector


p =

   1.0e+04 *

         0
         0
   -1.1200


k =

  -2.2400e+06


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf

## Closed Loop- Negative feedback with Controller (D)

sys = tf([(-k)*A,0],[1,k,(-k)*A])
figure(4);
subplot(3,1,1);
impulse(sys);
title('Impulse Input');
subplot(3,1,2);
step(sys);
title('Step Input');
[z,p,k] = tf2zp([-k*A,0],[1,k,(-k)*A])
subplot(3,1,3);
zplane(z,p);
S = stepinfo(sys)

sys =
 
          4.48e08 s
  -------------------------
  s^2 - 2.24e06 s + 4.48e08
 
Continuous-time transfer function.


z =

     0


p =

   1.0e+06 *

    2.2398
    0.0002


k =

   4.4800e+08


S = 

  struct with fields:

        RiseTime: NaN
    SettlingTime: NaN
     SettlingMin: NaN
     SettlingMax: NaN
       Overshoot: NaN
      Undershoot: NaN
            Peak: Inf
        PeakTime: Inf

## Closed Loop- Positive feedback with Controller (D)

sys = tf([(-k)*A,0],[1,k,k*A])
figure(5);
subplot(3,1,1);
impulse(sys);
title('Impulse Input');
subplot(3,1,2);
step(sys);
title('Step Input');
[z,p,k] = tf2zp([-k*A,0],[1,k,k*A])
subplot(3,1,3);
zplane(z,p);
S = stepinfo(sys)

sys =
 
         -8.96e10 s
  -------------------------
  s^2 + 4.48e08 s + 8.96e10
 
Continuous-time transfer function.


z =

     0


p =

   1.0e+08 *

   -4.4800
   -0.0000


k =

  -8.9600e+10


S = 

  struct with fields:

        RiseTime: 0
    SettlingTime: 0.0196
     SettlingMin: -199.9633
     SettlingMax: -0.2598
       Overshoot: Inf
      Undershoot: Inf
            Peak: 199.9633
        PeakTime: 9.2103e-07

## Math Analysis

Independent: Time(t) Dependent: Mass(M) Constant: Non-zero constant(A), Constant(A)

## Comparison Analysis

1) System without controller behaves exactly like an exponential decay with the system decaying exponentially.

2) On adding a proportionality controller to system, the system becomes unstable.

3) On adding a Integrator controller to system, the response times have decreased hugely, making the system reach stability faster than a P controller.

4) Integrator controller adds a pole to zero also.

5) On addition of a differentiator controller in negative feedback the system becomes unstable.

6) A zero gets added at origin due to the differentiator.

7) On addition of a differentiator controller in positive feedback the system becomes stable.

# 4(a) 1st Order Differential Equation Model

This was done in Simulink of MATLAB R2020b.

# 4(b) 2nd Order Differential Equation Model

This was done in Simulink of MATLAB R2020b.

# 4(c) Exponential Decay- Radioactive Material Model

This was done in Simulink of MATLAB R2020b.

# 5(a) Migration of Scripts to GNU Octave

The results matched with the scripts executed in the MATLAB R2020b

GNU Octave is software featuring a high-level programming language, primarily intended for numerical computations. Octave helps in solving linear and nonlinear problems numerically, and for performing other numerical experiments using a language that is mostly compatible with MATLAB.

# 5(b) Migration of Model to OpenModelica

The results matched with the models executed in Simulink of MATLAB R2020b.

OpenModelica is a free and open source environment based on the Modelica modeling language for modeling, simulating, optimizing and analyzing complex dynamic systems. This software is actively developed by Open Source Modelica Consortium, a non-profit, non-governmental organization.